E.2	Conduct a walk-through to detect opportunities for improving energy efficiency through operational and equipment adjustments

A simple walk-through of the property can help identify issues and identify opportunities for improvement in energy systems.

To be eligible for this point

- Use the checklist to conduct a walk-through of the property, acting on any issues and opportunities uncovered in the inspection

- Plan to use the checklist to conduct regular inspections as part of the property’s preventive maintenance program

To claim this point

- Submit the completed checklist or an equivalent inspection checklist with the application

| Property energy efficiency walk-through |  |  |  |  |
| --- | --- | --- | --- | --- |
| Walk-through conducted by |  |  |  |  |
| Date(s) |  |  |  |  |
| Date(s) of last walk-through |  |  |  |  |
| Operations & maintenance (O&M) |  |  |  |  |
| Item | OK | Not OK | Notes or action required |  |
| Preventive and routine maintenance plan/schedule adhered to |  |  |  |  |
| Benchmarking tool (e.g., Portfolio Manager®) regularly updated |  |  |  |  |
| EMS/BAS/BMS functioning and fully engaged |  |  |  | Example: Contact BAS vendor to check that all features are fully utilized |
| Equipment room(s) orderly |  |  |  |  |
| Vacant units appropriately conditioned/lighted |  |  |  |  |
| Maintenance team trained on building equipment |  |  |  |  |
| Contractors effective and fully utilized |  |  |  |  |
| Thermostats adjusted seasonally at optimal set-points |  |  |  |  |
| Access to thermostats appropriately restricted |  |  |  |  |
| Building envelope |  |  |  |  |
| Item | OK | Not OK | Notes or action required |  |
| No visible damage on exterior walls |  |  |  |  |
| Separation between metal-panel skin and structural steel to provide a thermal break |  |  |  |  |
| Fully insulated partitions at marketing towers |  |  |  |  |
| Doors properly aligned and sealed |  |  |  |  |
| Automatic and vestibule doors functional |  |  |  |  |
| Windows properly aligned and sealed |  |  |  |  |
| Blinds and curtains fully functional |  |  |  |  |
| No windows and/or doors left open |  |  |  |  |
| Window tinting or film installed |  |  |  |  |
| Conditioned and unconditioned spaces properly insulated |  |  |  |  |
| Outdoor shading devices properly positioned and functional |  |  |  |  |
| Roof properly maintained and in good condition |  |  |  |  |
| Cool/reflective roof installed |  |  |  |  |
| HVAC |  |  |  |  |
| Item | OK | Not OK | Notes or action required |  |
| HVAC system meets loads |  |  |  |  |
| HVAC SEER ratings of 15+ |  |  |  |  |
| Equipment free of soot, mineral deposits, pooling water, etc. |  |  |  |  |
| Filters and coils clean and maintained |  |  |  |  |
| Duct and pipe insulation in place and intact |  |  |  |  |
| Run-time schedule optimized/ HVAC operating only when necessary |  |  |  |  |
| Sensors functional and calibrated |  |  |  |  |
| Automatic controls functional and properly set |  |  |  |  |
| Dampers and valves close tightly |  |  |  |  |
| Steam traps maintained and functional |  |  |  |  |
| No unnecessary or mysterious cycling |  |  |  |  |
| Compressor system staging and sequencing optimal |  |  |  |  |
| Fans and pumps not operating excessively |  |  |  |  |
| Variable frequency drives (VFDs) operating as intended |  |  |  |  |
| Air flow feels correct and consistent from one area to the next |  |  |  |  |
| No noticeable air leaks from ducts and plenums |  |  |  |  |
| Return, outdoor, and exhaust dampers properly sequenced |  |  |  |  |
| Air inlets and outlets clean and unobstructed |  |  |  |  |
| Lighting |  |  |  |  |
| Item | OK | Not OK | Notes or action required |  |
| No lights on in unoccupied units |  |  |  |  |
| LEDs installed where possible |  |  |  |  |
| Occupancy sensors installed and functional |  |  |  |  |
| Lighting sweep switches installed and functional |  |  |  |  |
| Security/outdoor lighting automatically controlled and not excessive |  |  |  |  |
| Domestic hot water |  |  |  |  |
| Item | OK | Not OK | Notes or action required |  |
| Hot water temperature not excessive |  |  |  |  |
| Water temperature reduced during unoccupied periods |  |  |  |  |
| System insulation installed and intact |  |  |  |  |
| No apparent water drips or leaks |  |  |  |  |
| Water heater coils clean and maintained |  |  |  |  |
| Hot water recirculation pumps do not run continuously |  |  |  |  |
| Plug loads |  |  |  |  |
| Item | OK | Not OK | Notes or action required |  |
| Point-of-sale systems off after operating hours |  |  |  |  |
| Computers off after operating hours |  |  |  |  |
| Copiers off after operating hours |  |  |  |  |
| No space heaters in use |  |  |  |  |

Alternative documentation
Instead of this form, you may submit at least one of the following to IREM®:

- Copy of any checklist used for your audit